Isaac Sim 5.1 Warehouse Simulation Design Plan

Object-state-driven warehouse behavior inside a zone-based layout, explicitly adjusted for Isaac Sim 5.1 integration risk and Claude-assisted delivery.

• Purpose: define the simulation architecture before coding. • Platform scope: Isaac Sim 5.1, with simulator-facing APIs treated as version-sensitive. • Design principle: objects hold truth, rules drive behavior, zones measure outcomes. • Delivery intent: reusable single scene, multiple scenarios via state and timing changes.

# 1. Executive Summary

This project builds one reusable warehouse environment in Isaac Sim 5.1 with three physical areas—Shelves Area, Staging Area, and Loading Zone—and drives scenarios through explicit object states instead of relying on complex continuous autonomy from the start. The design keeps the scene stable while scenario behavior changes through object state, timing, and release logic.

Key design decision

Use zone-based geometry for physical layout and observability, but use object-state-driven logic for control. This separation makes the simulation easier to implement, easier to test, and safer to generate with Claude Code while still respecting Isaac Sim 5.1 version-specific APIs.

# 2. Isaac Sim 5.1 Design Constraints

• All simulator-facing code must be treated as Isaac Sim 5.1-specific until verified locally.

• Generated code may create architecture, state logic, and logging freely, but stage setup and simulator integration must be isolated behind adapter/helper modules.

• The first implementation should prefer deterministic waypoint motion over full autonomy or navmesh-heavy behavior.

• Python-first control is preferred; GUI-only workflows should be used sparingly and validated in the installed 5.1 build.

| Topic | Definition |
| --- | --- |
| Safe to design up front | Object models, state machines, rule engine, metrics schema, scenario presets, and repository structure. |
| Must be version-verified in 5.1 | Stage creation, extension wiring, physics stepping, prim helpers, transforms, sensors, and any vehicle or articulation control interfaces. |
| Recommended implementation bias | Use thin Isaac adapter modules so any 5.1-specific breakage is fixed once instead of across the whole codebase. |

# 3. Simulation Objective and Scope

| Topic | Definition |
| --- | --- |
| Objective | Build a simplified but operationally useful warehouse simulation for alert testing, queue behavior, buildup analysis, idle detection, congestion analysis, and proximity monitoring. |
| Included in v1 | Three fixed zones, 3–5 forklifts, pallets as tracked entities, one loading door, queue slots, waypoint routing, dwell timers, state transitions, zone metrics, and scenario presets. |
| Not required in v1 | Full warehouse management integration, high-fidelity autonomy, dynamic traffic planning, or photoreal scene complexity beyond what supports behavior testing. |

# 4. Physical Warehouse Layout

The environment should use one clear operational flow: Shelves Area → Staging Area → Loading Zone → return or exit. The physical layout should be simple enough to debug quickly and restrictive enough to create realistic queues and choke points.

| USD zone path | Purpose |
| --- | --- |
| /World/Zones/ShelvesArea | Rack and aisle zone where forklifts perform pickup/drop activities and where aisle traffic conditions can be triggered. |
| /World/Zones/StagingArea | Midstream buffer zone between shelves and dock. Supports waiting, accumulation, and controlled release. |
| /World/Zones/LoadingZone | Final approach and service area at the dock. Supports single-forklift service plus nearby queue positions. |

• Add support prim groups such as /World/Waypoints, /World/Forklifts, /World/Pallets, /World/QueueSlots, and /World/Metrics.

• Use explicit queue positions near the dock rather than ad hoc waiting behavior.

• Design aisle widths and turning radii for believability first; do not let automation decide geometry.

# 5. Core Object Model

| Object | Key state | Why it matters |
| --- | --- | --- |
| LoadingDoor | is_open | A single state can trigger loading pause, dock queue, and upstream buildup. |
| LoadingZone | active_forklift_id, queue_count, capacity, is_blocked | Represents dock occupancy and waiting pressure. |
| StagingArea | forklift_count, pallet_count, capacity, is_blocked | Supports buildup detection and release control. |
| ShelvesArea | forklift_count, active_task_count, is_congested | Supports aisle traffic and idle/congestion logic. |
| Forklift | state, current_zone, has_pallet, assigned_task, target_waypoint | Primary moving actor driven by a simple operational state machine. |
| Pallet | location, assigned_forklift_id | Separates material state from forklift state and avoids ambiguity. |
| QueueSlot / DockSlot | occupied_by, slot_type | Makes queue assignment deterministic and observable. |

# 6. Rule Engine and Behavior Flow

The rule engine decides what can happen next based on object state. Movement only executes a decision that has already been made. This keeps the logic readable and reduces debugging complexity in Isaac Sim 5.1.

• If the loading door is closed, no forklift may enter LOADING state.

• If the dock slot is occupied, the arriving forklift goes to the next queue slot or remains in staging.

• If staging is full, upstream forklifts must wait before entering.

• If a forklift has a pallet and a legal downstream action exists, it may advance to the next zone.

• Pallet location and forklift cargo state must always agree.

| Topic | Definition |
| --- | --- |
| Default forklift state ladder | IDLE → PICKUP_AT_SHELVES → MOVE_TO_STAGING → WAIT_IN_STAGING → MOVE_TO_LOADING → WAIT_AT_DOCK_QUEUE → LOADING → RETURNING |
| Default movement flow | Shelves Area → Staging Area → Loading Zone → return path or loop reset |
| Failure patterns to observe | Spawn collisions, invalid state transitions, queue deadlock, unrealistically tight spacing, and metrics mismatching visible behavior. |

# 7. Scenario Mapping

| Scenario | Primary levers | Expected area impact |
| --- | --- | --- |
| Dock Queue | Longer loading time, dock capacity = 1, faster arrivals | Queue forms in Loading Zone and pressure rises upstream. |
| Loading Pause | LoadingDoor.is_open = False or LoadingZone.is_blocked = True | Forklifts stop advancing to service and queues accumulate. |
| Area Build-Up | Staging inflow > outflow | StagingArea occupancy and dwell increase. |
| Aisle Congestion | Shared aisle timing, low speed thresholds, choke point use | ShelvesArea speeds drop and local occupancy rises. |
| Vehicle Idle | No downstream action or no task available | Forklift remains waiting or idle in staging or shelves. |
| Safety Proximity | Distance threshold breaches in dock or aisles | Event log captures near-neighbor duration and affected zone. |

# 8. Monitoring and Metrics

• Per zone: forklift count, pallet count, average speed, occupancy over time, dwell totals, and transition counts.

• Per forklift: current state, current zone, has_pallet, time in state, time in zone, and nearest-neighbor proximity.

• Per event: door open/close, queue formed, buildup threshold exceeded, state hold too long, pallet transfer, and proximity alert.

Metrics principle

Zones are the measurement layer; objects are the control layer. This division should remain visible in the implementation so the simulation can be reasoned about without stepping through every movement frame.

# 9. Implementation Phases

| Phase | Primary output | Isaac Sim 5.1 note |
| --- | --- | --- |
| 1. Version verification | Confirm 5.1 extension names, stage patterns, physics loop hooks, and asset availability. | Must be done manually before trusting generated simulator-facing code. |
| 2. Scene skeleton | Build floor, racks, staging area, dock, queue slots, and waypoints. | Use adapter layer for stage and prim creation. |
| 3. Object-state model | Implement LoadingDoor, areas, forklifts, pallets, and slots as data/state objects. | Safe for broad generation. |
| 4. State machine + rules | Implement forklift transitions, release logic, and queue handling. | Safe for broad generation; validate runtime behavior visually. |
| 5. Monitoring + logs | Zone monitors, event logging, and CSV or JSON metrics. | Safe for broad generation. |
| 6. Scenario presets | Parameterize Dock Queue, Loading Pause, Area Build-Up, Aisle Congestion, Vehicle Idle, Safety Proximity. | Keep presets data-driven. |
| 7. Validation | Compare visible behavior against metrics and thresholds. | Must be manual in Isaac Sim 5.1. |

# 10. Repository Architecture

Keep the simulator-facing layer narrow and isolate 5.1-specific calls. The rest of the project should operate on ordinary Python objects and rules.

• config/: layout, object defaults, scenario presets

• models/: object state classes for door, zones, forklift, pallet, and slots

• logic/: state machine, rule engine, loading manager, staging manager, pallet flow

• scene/: stage builder, zones, waypoints, Isaac Sim adapters

• monitoring/: zone monitor, metrics logger, event logger, proximity monitor

• output/: logs and metrics artifacts

# 11. Design Approval Checklist

• Physical zone layout is fixed and believable.

• Queue slots and staging hold positions are explicit.

• Object state definitions are finite and unambiguous.

• Rule engine covers door closure, dock occupancy, staging buildup, and pallet consistency.

• Isaac Sim 5.1 integration points are isolated behind helpers.

• Scenario presets change behavior without changing scene geometry.

• Metrics capture what operators actually need to review.